Governor Construction Plan

# Fixed Base

## Initial Components

- Cut 3/8” thick vertical pieces and machine ends square

  - Mark out then drill and tap

    - M3 (2.5mm)

    - 5BA (No. 36)

- Cut bottom 1/8” piece about 2” over-length.

  - Lightly fly-cut what will be the upper face. Ensure there is good support over the whole length and that the piece is clamped parallel to table motion.

  - Find approx centre (do not punch it) and markout then

    - drill & countersink clearance holes for M3 CSK fixings

    - drill clearance holes for 5BA fixings (No.30)

- Cut top 1/8” piece slightly over-length

  - Lightly flycut what will be the bottom face

  - Find centre on top face and mark out then

    - Punch centre

    - drill clearance holes for 5BA Studs

- Assemble the parts with the top 1/8” piece centre-punch on the outer face

- File away most of the excess length of the top 1/8” piece

- File/drawfile the long sides to remove any tiny steps between horiz and vert pieces (this can be done after the turning/boring opps)

## Turning and Boring

- Mount on faceplate, centring the top face punch-mark with a wiggler. Ensure it will be possible to turn the 3/8” side pieces most of the way down to the base (the excess length of the bottom part is now explained by its utility as a mount point)

- Centre drill and bring tailstock centre to bear.

- Turn down the 3/8” side pieces until unbroken curves are just created. Slow speed!

- Drill out where already centre drilled, progressing up the drill sizes and going right through base as well. The top hole will require a boring bar but this may also be advisable for the bearing hole in the base

- Open out top hole with boring bars.

- Turn a nice flat seating for the fixed collar

- Use a TCMT LH tool to cut into the base about half-way through at what will be their final length.. This is at variance to drawings.

- Remove from faceplate and cut off excess from 1/8” base piece then file down to the arc created in the previous step.

## Fixed Collar

- Stock in 3 jaw

- Brighten up the outside, turn down the step to an interference fit in the already-made 1/8” plate and face off to step-length

- Centre drill and drill. NB make it a close 3/16” only for about (what will become the topmost) ¼” and drill clear for most of the length. Consider reaming by hand for 3/16”.

- Reverse in chuck and face off to a smooth finish. Chamfer.

- Press the collar into the 1/8” plate (dis-assembled from other components). Take care NOT to bend the 1/8” plate while pressing.

## Drive Entry

- Machine the collar for drive entry. The threading is ME and don’t forget to undercut thread where the die cannot cut. The 5/16” (approx) projection on the inner end will probably need to be trimmed on assembly to keep the incoming bevel gear correctly positioned (maybe include a thrust washer too?)

- Mark off one of the 3/8” vertical pieces at the point of drive entry

  - Centre mark, clamp in mill/drill vice and lock table.

  - Centre drill and drill to size for threading

  - Mill flat with slot drill to make a ½” seating. If the brightened ½” stock will fit nicely into a plunged seating then do it that way, otherwise cut a channel right across.

  - Thread 3/8” ME

# Moving Parts

## Ball and Lifting Arms

Symmetry matters! Initially drill pairs of over-length stock near to each end and pin with 1/16” to keep relative positions during the following operations. All the following operations are conducted with pinned pairs. (Use toolmakers clamp to allow the pair to be flipped over and set to the same x-y position in the milling vice: tighten the clamp with it butting firmly up to vice before loosening the vice and, after turning the workpiece over, butt the clap up to the vice before retightening it)

- Making 1/16” tangs and centre-narrowing (size them to fit a slotting saw trial cut in some scrap)

  - The design is such that the hex rod flats can be clamped in the machine vice and a ½” slot drill run through.  For the (end) tangs, leave surplus stock on the end to provide vertical support while milling – i.e. mill this as a slot and later remove the excess.

  - Use a parallel piece between the vice jaws to rest the hex rod on with the part to be machined protruding from the vice. Hold the work between the main jaws of the vice and not the top step so that the hex rod can be properly supported by the parallel.

  - For the first tang, mill from both sides down to the same vertical mill-head height but OVER-SIZED. Measure and calculate additional milling.

- Drilling

  - Is obvious. NB the slot end of the lift arms has a small flat filed off the hex rod to allow a good drill-start. The hole separation is not critical but MUST be the same in both arms.

- Making 1/16” slot

  - Use a 1/16” slotting saw (same one as for top and bottom boss, below). Run a trial to ensure the slot is centred.

  - Use a parallel as before

- Turn down and tap (4BA) the ball arms.

## Top and Bottom Boss

Should probably interleave Top and Bottom Boss operations

- Bottom Boss.

  - File/Mill end of stock square

  - Use slitting saw to cut 1/16” slot.

  - Mark out and drill 1/16” holes (or use edge finder)

  - Cut off and file/mill to length

  - Mount in 4 jaw, using DTI to centre. Ensure enough tool-clearance for step (g)

  - With a ¼” slot drill in tailstock drill-chuck, bore down ¼” (to base of 1/16” slot)

  - Without removing work from 4-jaw (concentricity!), turn 5/16” for running collar, slightly under the target length of the collar

  - Drill 3/16” ensuring close fit to rod (No 13 drill gives an easy fit)

  - Part off and press into running collar

- Top Boss

  - Mount in 4 jaw, using DTI to centre

  - Face off and drill 3/16” ensuring close fit to rod

  - Use slitting saw to cut 1/16” slot.

  - Mark out and drill 1/16” holes

  - Cut off and file/mill to length

  - Create 60deg shoulders with dremel.

## Running Collar

- Face off

- Centre drill and drill out to size progressively. Aim undersize initially and use a hole gauge to compare size to bottom boss mating part.

- Turn down to 7/8” OD. (or slightly different OD to suit chamfer on collar made earlier)

- Cut the 3/16” wide channel using low speed (250 RPM) and a piece of 3/16” square tool steel. (I had milled a ½” slot in some 3/16” brass flat as a gauge for final depth of cut

- Lightly chamfer or file down the corners

- Part off

## “Balls”

The absolute size isn’t important but balance is so take care with measurements. E.g. OD should be the same for both, so should shoulders and so should the drilling depths.

- Set top slide over to 45 degrees

- Brighten OD and face off stock.

- Centre drill, drill No. 31. Drill shallow 9/64”section to accommodate unthreaded portion of ball-arm.

- Tap 4BA

- Use topslide to turn main 45 deg shoulder

  - (suggested method to  achieve symmetry between balls)

  - Set top slide to a starting position from which it will be able to advance across whole of shoulder

  - Set cross-slide so cutting tool just touches the (brightened) stock OD (use mark blue) about 1/8” in from faced end and zero the collar.

  - (machine off) advance cross-slide 1/8” and position saddle so that the cutting tool just contacts the faced-off end and lock saddle.

  - Back off the cross-slide to its “zeroed” position. This is the starting point for machining.

  - Turn the shoulder by traversing the top slide then advancing the cross-slide up to a max cross-slide advance of ¼”.

- Part off or saw off (I parted)

- Face off to neaten (actually to ensure both are same length)

- Create small shoulder as before.

## Hand Crank

- Make the handle

  - Drill (4mm) out a 5/8” length sleeve and face the ends. I used brass tube stock so minimal stock to drill-out.

  - Turn down a M4 nut to make a narrow lock-nut. Mount on threaded rod with a washer bearing onto chuck jaws. Chamfer the corners as a normal nut

  - Using a standard M4 machine screw (I had cross-head ones). Turn down to ¼” diam head to match sleeve and reduce the head to about 1/16” thickness. Lightly drill-out the remains of the cross-drive in the head for appearance.

- Make the crank “throw”

  - Mark out, including radius ends.

  - Drill and thread – its obvious! (No.13 drill for the “3/16” shaft)

  - The 8BA threaded hole is for a lock-screw onto the shaft. Put a small flat on the shaft to seat this.

## Other  Notes

- The bevel gears will need to be bored out to 3/16” (assuming Meccano 26teeth bevel gears are used.

- 3/16” brass rod is used for the input drive and governor spindle. Determine length by setting up. Mark position of top boss then disassemble and drill through top boss for the 1/16” (middle) pin. It probably makes sense to set the governor up so that the “running  collar” doesn’t rest on the “fixed collar” (so long as the top boss has symmetrical shoulders on it’s slot)

- 1/16” pins are used for the pivot points

  - For the top/bottom bosses, cut about 3/64” + 5/16” and centre-punch the ends while the pin is in-situ so that the pin will not slide out

  - For the arm-centre pivot approx 1/16” lengths of rubber sleeve was pushed over the pins to hold them in place

# Copyright & Licence

This work is © Adam Cooper, 2010, and licensed for use under Creative Commons Attribution terms: Attribution 2.0 UK: England & Wales.

This is to ensure a tight butt between the vertical and horizontal members

A No. 14 drill seemed best for steel whereas No.13 gave a nice fit in brass.

I just used a blank bit (with a 20 deg front clearance) quickly polished up and without any side clearance. This was fitted in the top slide and squared to the side of the chuck body but during turning the top slide migrated towards the chuck by a few thou so the channel didn’t come out completely perp to the central axis. Hope it doesn’t matter as I can’t seem to correct it now (maybe some side clearance would have helped).