# Towers of Hannoi

© Adam Cooper, December 2008
Licensed: Creative Commons Attribution 2.0 England

## Base (Aluminium)

- Mill the sides of the base to size. Don’t worry about being slightly under 2” across.

- Mark out the 3 holes in the base (I used Jenny oddlegs at 11/16”), punch

- Drill and tap M3 in the drilling machine to get nice and square

- Mount on lathe in 4-jaw. 2 jaws will need to be reversed. Needs care to get it roughly parallel.

- Lightly countersink and then counterbore 3mm. This is to allow for the fact that the threading on the pillars will be incomplete. The countersink helps the drill to locate nicely.

## Pillars (Brass)

- Chuck 3/16” rod in the lathe with minimum required projecting, face off and turn down slightly over ¼” to 3mm. I used a lathe end stop and piece of ¼” stock to set this up.

- Thread M3 using a tailstock die holder

- Remove from chuck and hacksaw to size (or pull rod out to required length and part off)

- Face top of pillar to length and chamfer 45 degrees. It makes sense to do a this last step for all three after completing steps 1-3 for all.

## Disks (Aluminium)

- Face off

- Chuck in lathe and brighten up outer diam for 3/8”

- Very lightly chamfer to remove burr

- Drill 3/16” to ¼”. I didn’t bother to centre drill (unusually)

- Part off disk to 3/16”. I used the lathe end stop again and actually parted to about 7/32”. After starting the part, lightly chamfer the bit to be parted off where the parting blade has forced up a burr.

- Very lightly countersink the face where the drill entered. This will be the top face.

- More deeply countersink the other face as this will be the entry point when in use. I fine-filed this face as it had parted slightly convex and then used emery 320 grit.

## Finishing Off

The top face of the base was left as-turned as it has a nice concentric CD-like finish but the sides were 320 emery’ed. The base of the base was just filed flat.